| Wastewater Management Option | Typical Effluent Total Nitrogen (mg/L) | Typical Effluent Total Phosphorus (mg/L) | Typical Nitrogen Reduction | Typical Phosphorus Reduction | Comments |
| --- | --- | --- | --- | --- | --- |
| Conventional Septic System | 40–70 | 8–15 | 10–30% | Little P removal after soil saturation | Baseline system; nitrogen readily reaches groundwater; no longer permitted within the watershed of the Indian River Lagoon; Florida Legislature set a 2030 deadline for their phase out. |
| Enhanced Nitrogen-Reducing Septic (Performance-Based Treatment System) | 10–20 | 5–10 | 65–85% | Little P removal after soil saturation | Performance-based or nitrogen-reducing septic technologies provide enhanced nitrogen removal and pathogen treatment beyond conventional septic systems. |
| Conventional Septic + In-Ground Nitrogen-Reducing Bioreactor | 15-20 | 8–15 | 65–75% | Little P removal after soil saturation | Passive subsurface treatment systems installed beneath septic drainfield to further reduce nitrogen before wastewater reaches groundwater or adjacent water bodies. |
| Distributed (Decentralized) Sewer* | 10-20 | 8–15 | ≥80% | Little P removal after soil saturation | Individual on-site systems with remote, professional operation and maintenance. |
| Private Package Treatment Plant | 10-30 | 8–15 | 50–80% | 80–95% | Similar performance to distributed sewer; shared treatment facilities serving multiple homes or developments where individual onsite systems are unsuitable and centralized sewer is not available. |
| Central Sewer (Advanced Wastewater Treatment) | 2-3 | 0.1–1 | 90–95% | 90–99% | Highest level of treatment |

* According to Florida Statute 403.814, “Distributed wastewater treatment unit” or “DWTU” means an advanced onsite closed-tank wastewater treatment system that is remotely operated and controlled by the permittee using an electronic control system and designed to achieve secondary treatment standards and a minimum of 80 percent total nitrogen removal before discharge to a subsurface application system.